CS_323

HIGH PERFORMANCE MICROPROCESSORS

(Attempt 2 questions out of 3)

# Question 1

(a) Describe how a 1-bit dynamic RAM cell operates. Your answer should include a description of the process of reading and writing to the RAM cell. 									             [5 marks]

(b) Describe the structure of a dynamic RAM chip. Your answer should include an explanation of the process of reading first a row (Row Address Strobe) then a column (Column Address Strobe).					 [5 marks]

(c) The structure of a dynamic RAM chip is such that after one bit has been read, it is relatively simple to quickly read other, nearby, bits.  Explain how this works.									 [5 marks]

(d) Consider a machine with 32-bit memory addresses, and a cache with a total size of 64Kbytes, in which each cache slot can hold eight bytes. What are the sizes of the word, slot and tag fields for:

(i) a direct-mapped cache;

(ii) a two-way set associative cache; and

(iii) and a fully associative cache.

What would be the effect of changing the memory address size to 64 bits, and keeping the cache the same size.					           [10 marks]

# Question 2

(a) Pipelined processors attempt to overlap the execution of multiple, sequential instructions. To do this, they must resolve a number of dependencies that, if ignored, would disrupt execution and possibly result in erroneous results. Explain, perhaps with the aid of simple examples, the dependencies that apply to pipelined processors only. 					[5 marks]

(b) One of the dependencies in your answer to (a) is inherent in the sequential nature of programs. Unlike the other dependencies, in all currently-available microprocessors this particular dependency always causes a pipeline stall. Which dependency is this? In principle, pipeline stalls caused by this dependency could be removed, in at least some cases: how?		[5 marks]

(c) Consider the following pipeline.

Construct the reservation table, determine the forbidden latencies, and hence derive the collision vector. 							[5 marks]

(d) In principle, a pipeline such as that shown in (c) should be able to complete two operations in four cycles. How can you infer this from the reservation table? How would you determine, from the reservation table, if this were actually the case? What steps would you take to modify a pipeline in order to ensure that it reached the theoretical bound on throughput? Note: you are not being asked to actually determine the minimum latency cycles.		  							           [10 marks]

# Question 3

(a) Consider a pipelined machine with a reorder buffer, a bank of user registers, a Level 1 data cache, and main memory. Explain the path of the result of an instruction once the execution phase has finished in the following circumstances.

- An arithmetic operation where the result is to be stored in a register.

- A memory write operation with a write-through cache.

- A memory write operation with a write-back cache.

- An arithmetic operation where the result is to be stored in a register, and used by a closely-following instruction.

[10 marks]

(b) What is a branch target buffer, and what information does it minimally contain? What information could you add to speed up instruction execution? What other branch-related information could be usefully stored in a branch target buffer?									 [5 marks]

(c) A typical branch history mechanism using two bits of history information requires two consecutive mis-predictions before it changes from predicted taken to predicted not taken, and vice versa. Why is this?			 [5 marks]

(d) With simple branch history mechanisms, there is little gain in having more than two history bits. Explain how, and why, extra history bits can be usefully exploited by a correlating predictor. You may wish to illustrate your answer with an example of a (1,1) correlating predictor.			 [5 marks]