CS_323

HIGH PERFORMANCE MICROPROCESSORS

(Attempt 2 questions out of 3)

# Question 1

(a) Describe the seven characteristics commonly used to characterize RISC architectures. List the three processors that arguably started the RISC movement, with brief notes on their background. 									             			           [10 marks]

(b) Explain why it is generally easier to build pipelined and/or superscalar implementations of RISC processors than CISC processors.				 									[5 marks]

(c) Intel’s IA-32 architecture is not RISC yet they have successfully built pipelined and superscalar implementations (as have AMD). Explain the strategy they have used. You should mention any disadvantages.

[5 marks]

(d) AMD have publicly stated that they believe that RISC architectures no longer offer a performance benefit over CISC. Explain why this may be true.							 			[5 marks]

# Question 2

(a) Consider the following pipeline.

Construct the reservation table, determine the forbidden latencies, and hence derive the collision vector. 							[5 marks]

(b) The minimum latency cycles of the pipeline in (a) are such that it theoretically should be able to complete four operations in eight cycles. How would you determine, from the reservation table, if this were the case? What steps would you take to modify a pipeline in order to ensure that it reached the theoretical bound on throughput? Illustrate your answer with a simple example. Note: you are not being asked to actually determine the minimum latency cycles.							           [10 marks]

(c) The pipeline example in part (a) is concerned with using the same pipeline stage multiple times for the same operation. A related concept is Intel’s hyperthreading, which attempts to use the same pipeline simultaneously for different and unrelated operations. What is the rationale behind the introduction of hyperthreading and under what circumstances would you expect to see performance gains?					           [10 marks]

# Question 3

(a) A Very Long Instruction Word, or VLIW, machine has, as the name suggests, very long machine instruction words. Each machine instruction word can encode a number of operations, all of which are simultaneously executed.

- Compare and contrast VLIW and superscalar machines. What are the main advantages and disadvantages of a VLIW machine in this context?

- Explain, perhaps with a simple example, why it is possible for one operation within a VLIW instruction word to be forced to stall, while at the same time the other operations are permitted to continue.

- There are two options for handling the situation in (ii), one of which results in higher performance, and the other in simpler hardware. What are these two options?

[10 marks]

(b) Intel have attempted to address some of the usual VLIW problems in their IA-64 architecture. Describe in as much detail as you can the structure of IA-64 instructions. Your answer should include discussion of groups, bundles, stops and templates.							           [10 marks]

(c) IA-64 includes 64 1-bit predicate registers. Explain, perhaps with a simple example, how these predicate registers are used.			[5 marks]